# Moxa x86 Linux Software User Manual

x86 Linux Drivers and SDK

Exported on 2024-11-12 18:29:48

Table of Contents

1	1. Introduction	6

1.1	Available Model Series	6

2	2. Getting Started	7

2.1	Linux OS Installation Instructions	7

2.1.1	Prepare bootable USB drive	7

2.1.2	Current Supported Distributions	7

2.1.3	How to Enter BIOS Menu	7

3	3. x86 Linux SDK Wizard	8

3.1	Basic Information	8

3.2	Moxa x86 Linux SDK Applicable Table	8

3.3	Software Flow Diagram	9

3.3.1	Software Diagram	9

3.3.2	User Interface	9

3.3.3	Notice Before Starting Installation	10

4	4. Peripheral Interface Operations	11

4.1	Utilities	11

4.1.1	Utilities Applicable Table	11

4.1.2	Serial Port	12

4.1.3	Digital IO (DIO) Port	13

4.1.4	LTE (mPCIe slot) Module Control	13

4.1.5	Programmable LED Control	14

4.1.6	Relay Port State Control	15

4.1.7	Power Input Port State	15

4.1.8	USB Port Power State Control	16

4.1.9	Scaler Utility	17

4.1.10	MCU upgrade tool	17

4.1.11	Moxa MCU Manager	18

4.1.12	Moxa Disk Hotswap Daemon	21

4.1.13	HSR/PRP Utility	22

4.1.14	IRIG-B Utility	23

4.1.15	MCIM wrapper	25

4.2	Drivers	28

4.2.1	Drivers Applicable Table	28

4.2.2	moxa-it87-gpio-driver	29

4.2.3	moxa-it87-serial-driver	30

4.2.4	moxa-it87-wdt-driver	30

4.2.5	moxa-mxuport-driver	31

4.2.6	moxa-mxu11x0-driver	31

4.2.7	moxa-sdhci-pci-driver	32

4.2.8	moxa-gpio-pca953x-driver	33

4.2.9	moxa-hid-ft260-driver	33

4.2.10	moxa-irigb-driver	33

4.2.11	intel-gpu-i915-backports	34

4.3	Libraries	35

4.3.1	libgpiod	35

5	5. Basic Linux Concepts	36

5.1	5.1. Mounting SD Card Slot on BXP/DRP Series	36

5.2	5.2. Secure Boot	37

5.3	5.3. Linux PTP (IEEE 1588)	37

5.3.1	Example for Linux PTP setting up	38

6	6. Troubleshooting	41

6.1	6.1. How to Print Kernel Message from Linux Environment	41

6.2	6.2. How to Collect Systems Logs from Linux Environment	41

6.3	6.3. How to Get Installation Logs from Moxa x86 Linux SDK Install Wizard	41

6.4	6.4. How to Get Hardware Information on Host	42

7	7. Appendix	44

7.1	7.1. The License/Commercial-Use of Linux Distributions	44

7.1.1	Debian	44

7.1.2	Ubuntu	44

7.1.3	Red Hat Enterprise Linux (RHEL)	44

7.1.4	CentOS	45

8	8. Revision History	46

- 1. Introduction

  - Available Model Series

- 2. Getting Started

  - Linux OS Installation Instructions

    - Prepare bootable USB drive

    - Current Supported Distributions

    - How to Enter BIOS Menu

- 3. x86 Linux SDK Wizard

  - Basic Information

  - Moxa x86 Linux SDK Applicable Table

  - Software Flow Diagram

    - Software Diagram

    - User Interface

    - Notice Before Starting Installation

- 4. Peripheral Interface Operations

  - Utilities

    - Utilities Applicable Table

    - Serial Port

    - Digital IO (DIO) Port

    - LTE (mPCIe slot) Module Control

    - Programmable LED Control

    - Relay Port State Control

    - Power Input Port State

    - USB Port Power State Control

    - Scaler Utility

    - MCU upgrade tool

    - Moxa MCU Manager

    - Moxa Disk Hotswap Daemon

    - HSR/PRP Utility

    - IRIG-B Utility

    - MCIM wrapper

  - Drivers

    - Drivers Applicable Table

    - moxa-it87-gpio-driver

    - moxa-it87-serial-driver

    - moxa-it87-wdt-driver

    - moxa-mxuport-driver

    - moxa-mxu11x0-driver

    - moxa-sdhci-pci-driver

    - moxa-gpio-pca953x-driver

    - moxa-hid-ft260-driver

    - moxa-irigb-driver

    - intel-gpu-i915-backports

  - Libraries

    - libgpiod

- 5. Basic Linux Concepts

  - 5.1. Mounting SD Card Slot on BXP/DRP Series

  - 5.2. Secure Boot

  - 5.3. Linux PTP (IEEE 1588)

    - Example for Linux PTP setting up

      - [Ordinary Clock (OC) mode]

      - [Boundary Clock (BC) mode]

      - [Transparent Clock (TC) mode]

- 6. Troubleshooting

  - 6.1. How to Print Kernel Message from Linux Environment

  - 6.2. How to Collect Systems Logs from Linux Environment

  - 6.3. How to Get Installation Logs from Moxa x86 Linux SDK Install Wizard

  - 6.4. How to Get Hardware Information on Host

- 7. Appendix

  - 7.1. The License/Commercial-Use of Linux Distributions

    - Debian

    - Ubuntu

    - Red Hat Enterprise Linux (RHEL)

    - CentOS

- 8. Revision History

# 1. Introduction

This Moxa x86 Linux Software User Manual can help x86 Linux user to understand and navigate the usage of Moxa x86 Linux utilities and standard Linux operating system.

Below, we've provided comprehensive information for Getting Started, x86 Linux SDK wizard, Peripheral Interface Operations, Basic Linux Concepts, Troubleshooting and Appendix for x86 Linux user.

## Available Model Series

- BXP series

  - BXP-A100, BXP-C100, BXP-A101

- DRP series

  - DRP-A100, DRP-C100

- RKP series

  - RKP-A110, RKP-C110, RKP-C220

- V series

  - V3200

  - V3400

- MPC series

  - MPC-3000

- DA series

  - DA-820E

  - DA-820C

  - DA-682C

  - DA-681C

  - DA-680

# 2. Getting Started

The Getting Started section will introduce the Linux OS distribution installation instructions.

## Linux OS Installation Instructions

### Prepare bootable USB drive

At first, prepare a USB storage drive, download the Rufus to create bootable USB drive. Download the ISO image file and restore ISO image into USB storage drive.

### Current Supported Distributions

- Debian

  - Debian 11 (bullseye), Linux kernel 5.10

  - Debian 12 (bookworm), Linux kernel 6.1

  - Official Debian installation guide

- Ubuntu

  - Ubuntu 20.04 LTS (Focal Fossa), Linux kernel 5.4 (20.04.1), Linux kernel 5.15 (20.04.5), HWE kernel 5.15 or later version

  - Ubuntu 22.04 LTS (Jammy Jellyfish), Linux kernel 5.15 (22.04.3), Linux kernel 6.5 (22.04.4), HWE kernel 6.5 or later version

  - Official Ubuntu installation guide

- RedHat

  - RedHat 9, Linux kernel 5.14

    - Official RedHat 9 download link

    - Official RedHat 9 installation guide

- CentOS 7

  - CentOS 7.9, Linux kernel 3.10

    - CentOS-7-x86_64-DVD-2009.iso download link

### How to Enter BIOS Menu

Boot up device and press F2 key from keyboard to enter BIOS menu, and select boot from USB from UEFI mode.

Then follow the distribution's official installation guide to finish OS installation procedure.

# 3. x86 Linux SDK Wizard

## Basic Information

The Moxa x86 Linux SDK enables the easy deployment on the Moxa x86 IPC platform. The SDK contains components for peripheral drivers, peripheral control tools and configuration files.

It also provides deployment features, such as build & installation log, dry-run, and self test on target model. User can download the Moxa x86 Linux SDK zip file from official product's website.

Below is the list of files:

- *.tgz - the tarball file of x86 Linux SDK Install Wizard

- README.docx/README.md - the user manual of x86 Linux SDK Install Wizard

- sources_list - the list of source code

- build_info - build information

Please extract the tgz tarball file under Linux OS environment to avoid file permission issue.

## Moxa x86 Linux SDK Applicable Table

| - Model Name | - Available SDK Version | - Support Linux Distributions |
| --- | --- | --- |
| - BXP-A100/BXP-C100 | - V1.2 | - Debian 11/12, Ubuntu 22.04/20.04 LTS & HWE, RedHat 9, CentOS 7.9 |
| - DRP-A100/DRP-C100 | - V1.2 | - Debian 11/12, Ubuntu 22.04/20.04 LTS & HWE, RedHat 9, CentOS 7.9 |
| - RKP-A110/RKP-C110 | - V1.2 | - Debian 11/12, Ubuntu 22.04/20.04 LTS & HWE, RedHat 9, CentOS 7.9 |
| - V3200 | - V1.2 | - Debian 12, Ubuntu 22.04 LTS (HWE) |
| - V3400 | - V1.2 | - Debian 12, Ubuntu 22.04 LTS (HWE) |
| - MPC-3000 | - V1.2 | - Debian 11, Ubuntu 22.04 LTS, RedHat 9 |
| - DA-820E | - V1.2 | - Debian 12, Ubuntu 22.04 LTS, RedHat 9 |
| - DA-820C | - V1.2 | - Debian 12, Ubuntu 22.04 LTS & HWE, RedHat 9, CentOS 7.9 |
| - DA-682C | - V1.2 | - Debian 12, Ubuntu 22.04 LTS & HWE, RedHat 9, CentOS 7.9 |
| - DA-681C | - V1.2 | - Debian 12, Ubuntu 22.04 LTS & HWE, RedHat 9, CentOS 7.9 |
| - DA-680 | - V1.2 | - Debian 12, Ubuntu 22.04 LTS & HWE, RedHat 9, CentOS 7.9 |
| - RKP-C220 | - V1.2 | - Debian 12, Ubuntu 22.04 LTS (HWE), RedHat 9 |
| - BXP-A101 | - V1.2 | - Debian 12, Ubuntu 22.04 LTS (HWE), RedHat 9 |

## Software Flow Diagram

### Software Diagram

### User Interface

### Notice Before Starting Installation

- Please configure your network settings before installation

- To extract the tgz tarball file under Linux environment (e.g. tar xvf *.tar.gz)

- Run --dry-run option before installation, to check the target host device and environment are available

- Run --selftest option after installation, to check the status of drivers and tools

# 4. Peripheral Interface Operations

This guide is introduced the usage of Moxa peripheral interface control utility. These utilities should be installed after the x86 Linux SDK Wizard installation procedure.

User can check the status of utilities via running ./install.sh --selftest command.

## Utilities

### Utilities Applicable Table

| - Available Models | - Serial Port Utility | - DIO Port Utility | - PLED Utility | - Relay Utility | - Power Input State | - USB Power Utility |
| --- | --- | --- | --- | --- | --- | --- |
| - BXP-A100/BXP-C100 | - O | - O | - X | - X | - X | - X |
| - DRP-A100/DRP-C100 | - O | - X | - X | - X | - X | - X |
| - RKP-A110/RKP-C110 | - O | - O | - X | - X | - X | - X |
| - V3200 | - O | - O | - O | - X | - X | - X |
| - V3400 | - O | - O | - O | - X | - X | - X |
| - MPC-3000 | - O | - O | - X | - X | - X | - X |
| - DA-820E | - O | - O | - O | - O | - O | - O |
| - DA-820C | - O | - O | - O | - O | - O | - O |
| - DA-682C | - O | - O | - O | - O | - O | - O |
| - DA-681C | - O | - O | - O | - O | - O | - O |
| - DA-680 | - O | - O | - O | - O | - O | - X |
| - RKP-C220 | - O | - O | - X | - X | - X | - X |
| - BXP-A101 | - O | - O | - X | - X | - X | - X |

| - Available Models | - LTE (mPCIe slot) Module Utility | - Scaler Utility | - MCU Manager and upgrade tool | - HSR/PRP Utility | - IRIG-B Utility | - MCIM wrapper | - Disk Hotswap Daemon |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - BXP-A100/BXP-C100 | - X | - X | - X | - X | - X | - O | - X |
| - DRP-A100/DRP-C100 | - X | - X | - X | - X | - X | - O | - X |
| - RKP-A110/RKP-C110 | - X | - X | - X | - X | - X | - O | - X |
| - V3200 | - O | - X | - O | - X | - X | - O | - X |
| - V3400 | - O | - X | - O | - X | - X | - O | - O |
| - MPC-3000 | - X | - O | - O | - X | - X | - O | - X |
| - DA-820E | - X | - X | - X | - O | - X | - O | - X |
| - DA-820C | - X | - X | - X | - O | - O | - O | - X |
| - DA-682C | - X | - X | - X | - O | - X | - O | - X |
| - DA-681C | - X | - X | - X | - X | - X | - O | - X |
| - DA-680 | - X | - X | - X | - X | - O | - O | - X |
| - RKP-C220 | - X | - X | - X | - X | - X | - O | - X |
| - BXP-A101 | - O | - X | - X | - X | - X | - O | - X |

### Serial Port

The Moxa serial port mode control utility mx-uart-ctl, it is for getting and setting serial port's UART mode.

- Drivers dependency

  - moxa-it87-gpio-driver

  - moxa-it87-serial-driver

  - moxa-mxuport-driver

- Libraries dependency

  - libgpiod

Usage of UART mode control

Usage: mx-uart-ctl -p <port_number> [-m <uart_mode>] OPTIONS: -p <port_number> Set target port. -m <uart_mode> Set target port to uart_mode 0 --> set to RS-232 mode 1 --> set to RS-485-2W mode 2 --> set to RS-422 mode 3 --> set to RS-485-4W mode Example: Get mode from port 0 # mx-uart-ctl -p 0 Set port 1 to RS232 mode # mx-uart-ctl -p 1 -m 0

### Digital IO (DIO) Port

Moxa DIO port control tool mx-dio-ctl is for getting DI/DO and setting DO ports status (low/high).

- Drivers dependency

  - moxa-it87-gpio-driver

- Libraries dependency

  - libgpiod

Usage of DIO state control

Usage: mx-dio-ctl <-i¦-o <#port number> [-s <#state>]> OPTIONS: -i <#DIN port number> -o <#DOUT port number> -s <#state> Set state for target DOUT port 0 --> LOW 1 --> HIGH Example: Get value from DIN port 0 # mx-dio-ctl -i 0 Get value from DOUT port 0 # mx-dio-ctl -o 0 Set DOUT port 0 value to LOW # mx-dio-ctl -o 0 -s 0 Set DOUT port 0 value to HIGH # mx-dio-ctl -o 0 -s 1

### LTE (mPCIe slot) Module Control

Moxa LTE (mPCIe slot) module control tool mx-module-ctl is provided to control LTE power on/off state and SIM card select functions

- Drivers dependency

  - moxa-it87-gpio-driver

  - moxa-gpio-pca953x-driver

- Libraries dependency

  - libgpiod

Usage of LTE (mPCIe slot) module control tool

Usage: mx-module-ctl [Options] Operations: -s, --slot <module_slot_id> Select module slot -p, --power [on¦off] Get/Set power on/off module -r, --reset [on¦off] Get/Set reset pin to high(on)/low(off) to slot -i, --sim 1¦2 Get/Set sim card slot Example: Power on module 1 # mx-module-ctl -s 1 -p on Set module 2 reset pin to high # mx-module-ctl -s 2 -r on Select SIM 2 for module 1 # mx-module-ctl -s 1 -i 2 Get power status of module 1 # mx-module-ctl -s 1 -p Get current SIM slot of module 1 # mx-module-ctl -s 1 -i

### Programmable LED Control

Moxa LED control tool mx-led-ctl is provided to control programmable LEDs light on/off

- Drivers dependency

  - moxa-it87-gpio-driver

  - moxa-gpio-pca953x-driver

- Libraries dependency

  - libgpiod

Usage of programmable LED control tool

Usage: mx-led-ctl -i <led_index> [on¦off] OPTIONS: -i <led_index> Set LED index. Example: Get state from index 1 # mx-led-ctl -i 1 Set index 1 to on # mx-led-ctl -i 1 on

### Relay Port State Control

Moxa relay port state control tool mx-relay-ctl is for getting and setting relay ports status (NO: Normal Open/NC: Normal Closed).

- Drivers dependency

  - moxa-it87-gpio-driver

- Libraries dependency

  - libgpiod

Usage of relay state control

Usage: mx-relay-ctl -p <port_number> [-m <relay_mode>] OPTIONS: -p <port_number> Set target port. -m <relay_mode> Set target port to relay_mode 0 --> set to NC (Normal Closed) mode 1 --> set to NO (Normal Open) mode Example: Get mode from port 0 # mx-relay-ctl -p 0 Set port 0 to mode NC # mx-relay-ctl -p 0 -m 0 Set port 0 to mode NO # mx-relay-ctl -p 0 -m 1

### Power Input Port State

Moxa power input port state tool mx-input-power-state is for getting power input ports status (connected/disconnected):

- Drivers dependency

  - moxa-it87-gpio-driver

- Libraries dependency

  - libgpiod

Usage of power input port state tool

USAGE: mx-input-power-state -i <power_port> OPTIONS: -i <power_port> Get power input port state <connected/disconnected> EXAMPLE: Get power input port 0 state mx-input-power-state -i 0

### USB Port Power State Control

Moxa USB port power state control tool mx-usb-power-ctl is for setting/getting USB ports (front/rear/internal) power state (off/on) control:

- Drivers dependency

  - moxa-it87-gpio-driver

- Libraries dependency

  - libgpiod

Usage of USB power port state control tool

USAGE: mx-usb-power-ctl -i <usb_port> [-s <state>] OPTIONS: -i <usb_port> Get USB port power state 0: front 1: rear 2: internal -s <state> Set USB port power state 0: off 1: on EXAMPLE: Get USB front port power state mx-usb-power-ctl -i 0 Get USB rear port power state mx-usb-power-ctl -i 1 Set USB front port power state to off mx-usb-power-ctl -i 0 -s 0 Set USB internal port power state to on mx-usb-power-ctl -i 2 -s 1

### Scaler Utility

Moxa scaler utility is designed to configure basic settings of display devices, such as brightness, touch panel status, and OSD settings

Usage

Usage: mx-scaler-util [Options]... Options: -v, --version Get scaler firmware version -s, --status Get system status -m, --model Get model name -b, --brightness [0~10] Set brightness, query without arg -p, --touch-panel [0~1] Set touch panel on(1)/off(0), query without arg -o, --osd-config [0~1] Set OSD on(1)/off(0), query without arg

### MCU upgrade tool

The mx-lpc-mcu-upgrade-tool is a command-line utility designed for upgrading the firmware of MCU.

Warning

Before using MCU firmware upgrade tool, please stop Moxa MCU related serivce to avoid communication conflict issue.

Usage

Usage: mx-lpc-mcu-upgrade-tool [Options]... Options: -f, --file Start MCU upgrade from file -v, --version Get current MCU version Example: mx-lpc-mcu-upgrade-tool -f FB_MCU_V3000_V1.00S03_22060219.bin mx-lpc-mcu-upgrade-tool -v

### Moxa MCU Manager

Moxa MCU Manager (MMM) is used to control MCU(microcontroller) on Moxa x86 computer products. Including the LAN bypass, panel display, panel programmable LEDs, and update MCU ROM firmware.

Please ensure that mx-mcud  is running on background as daemon.

Usage

MOXA MCU Management Command-line Utility Usage: mx-mcu-mgmt [command] Flags: -h, --help Prints help information -v, --version Prints utility version Commands: mcu_version Get MCU firmware version relay Control relay mode wdt_reset Control watchdog reset mode wdt_relay Control watchdog relay mode poweroff_relay Control power off (S5) relay mode app_wdt_reset Control app watchdog reset mode app_wdt_relay Control app watchdog relay mode app_wdt_timout Control app watchdog timeout

LAN Bypass Modes

The LAN Bypass feature support the following three modes:

- [Connect]

  - LAN A and LAN B ports are connected to the NICs and is data transmitted through
system normally.

- [Disconnect]

  - LAN A and LAN B ports are neither connected to the NICs nor to each other,
which means that data packets are blocked.

- [Bypass]

  - LAN A and LAN B ports are connected to each other to keep transmitting data
without interruption even when a system device crashes or encounters a cyber attack.

Options of LAN Bypass Modes Control Utility

- Relay Mode

  - Description

    - Directly to get or set LAN Bypass mode from relay states.

  - Example

    - Get LAN Bypass Mode: mx-mcu-mgmt relay get_mode

    - Set LAN Bypass Mode:

      - Set Connect Mode: mx-mcu-mgmt relay set_mode connect

      - Set Disconnect Mode: mx-mcu-mgmt relay set_mode disconnect

      - Set Bypass Mode: mx-mcu-mgmt relay set_mode bypass

- Watchdog Reset Mode

  - Description

    - Use the Watchdog Reset Mode to indicate if the system needs to be reset after the MCU RTC watchdog (ds1374) is triggered

  - Example

    - Get Watchdog Reset Mode: mx-mcu-mgmt wdt_reset get_mode

    - Set Watchdog Reset Mode

      - Watchdog Reset OFF:  mx-mcu-mgmt wdt_reset set_mode off

      - Watchdog Reset ON:  mx-mcu-mgmt wdt_reset set_mode on

- Watchdog Relay Mode

  - Description

    - Use this Watchdog Relay Mode to indicate the relay mode to switch to after the MCU RTC watchdog (ds1374) is triggered.

  - Example

    - Get Watchdog Relay Mode: mx-mcu-mgmt wdt_relay get_mode

    - Set Watchdog Relay Mode

      - Set Watchdog Relay Connect Mode: mx-mcu-mgmt wdt_relay set_mode connect

      - Set Watchdog Relay Disconnect Mode: mx-mcu-mgmt wdt_relay set_mode disconnect

      - Set Watchdog Relay Bypass Mode: mx-mcu-mgmt wdt_relay set_mode bypass

- Power Off Relay Mode

  - Description

    - Use this Power Off Relay Mode to indicate the relay mode to switch to after the system is powered off (S5 state).

  - Example

    - Get Power Off Relay Mode: mx-mcu-mgmt poweroff_relay get_mode

    - Set Power Off Relay Mode

      - Set Power Off Relay as Disconnect Mode: mx-mcu-mgmt poweroff_relay set_mode disconnect

      - Set Power Off Relay as Bypass Mode: mx-mcu-mgmt poweroff_relay set_mode bypass

App Watchdog Modes Control Utility

The App Watchdog Modes Control Utility is to configure MCU’s behavior.

It provides set MCU timeout value, timeout-reset function, timeout-relay mode, and kicking service and daemon.

- APP WDT Mode

  - Description

    - Use this mode to enable or disable the MCU watchdog application

    - Activating the watchdog function is key to creating a trigger to activate LAN bypass when your application encounters issues or is unresponsive.

  - Example

    - Get APP WDT Mode and Timeout value: mx-mcu-mgmt app_wdt_timout get_timeout

    - Set APP WDT Mode and Timeout value:

      - Enable APP WDT Mode and set 10 sec timeout: mx-mcu-mgmt app_wdt_timout set_timeout 10

      - Disable APP WDT Mode: mx-mcu-mgmt app_wdt_timout set_timeout 0

- APP WDT Reset Mode

  - Description

    - Use this mode to indicate if the system needs to be reset after the MCU app watchdog is timeout triggered.

  - Example

    - Get App Watchdog Reset Mode: mx-mcu-mgmt app_wdt_reset get_mode

    - Set App Watchdog Reset Mode when app watchdog is triggered

      - Disable the reset system function: mx-mcu-mgmt app_wdt_reset set_mode off

      - Enable the reset system function: mx-mcu-mgmt app_wdt_reset set_mode on

- APP WDT Relay Mode

  - Description

    - Use this mode to indicate the relay mode to switch to after the MCU app watchdog is timeout triggered.

  - Example

    - Get App Watchdog Relay Mode: mx-mcu-mgmt app_wdt_relay get_mode

    - Set App Watchdog Relay Mode when app watchdog is timeout triggered

      - Set the App Relay mode

        - Set Connect Mode: mx-mcu-mgmt app_wdt_relay set_mode connect

        - Set Disconnect Mode: mx-mcu-mgmt app_wdt_relay set_mode disconnect

        - Set Bypass Mode: mx-mcu-mgmt app_wdt_relay set_mode bypass

### Moxa Disk Hotswap Daemon

Moxa Disk Hotswap Daemon is used to monitor the disk plug/unplug status with push buttons and programmable LEDs. User can remove disk via pressed push button to umount and remove disk safely.

This feature support on V3400 series.

- Libraries dependency

  - libgpiod

Add systemd service to use (if needed)

edit /lib/systemd/system/mx_disk_hotswapd.service

[Unit] Description=Moxa disk hotswap daemon service [Service] Type=oneshot ExecStart=/usr/sbin/mx-disk-hotswapd RemainAfterExit=yes [Install] WantedBy=multi-user.target

then enable service.

systemctl enable mx_disk_hotswapd.service

If user pressed push button over 3 seconds, the programmable LED will blinks 3 times and turn off light, and daemon will start to umount and remove target disk.

Alternatively, if pressed push button less than 3 seconds, the daemon will scan disk and mount target disk, the programmable LED will turn on to notify user the disk has been mounted, for example:

/dev/sda2 on /media/disk1p2 type squashfs (ro,relatime,errors=continue) /dev/sda1 on /media/disk1p1 type vfat (rw,relatime,fmask=0022,dmask=0022,codepage=437,iocharset=ascii,shortname=mixed,utf8,errors=remount-ro) /dev/sda3 on /media/disk1p3 type ext4 (rw,relatime)

### HSR/PRP Utility

Moxa HSR/PRP card utility is based on SMBUS to query FPGA related register.

Usage

[root@localhost moxa]# mxhsrprpd -h Usage: -h: Show this information. -B: Run daemon in the background -b: SMBUS device, default is /dev/i2c-0 -t: HSR/PRP Status update period. Default is 3 second. -m: configure to prp or hsr mode, default is prp mode. The argurement is [index]:[mode] [index] range from 0~7. [mode] 0 is prp, mode 1 is hsr. Ex: Set card 0 to hsr mode, card 1 to prp mode. root@Moxa:~# mxhsrprpd -t 2 -m 0:1,1:0 -s: configure fiber speed, default is auto detect mode. The argurement is [index]:[speed] [index] range from 0~7. [speed] 0 is 100M, 1 is 1000M. (default fiber speed is 1000M) Ex: Set card 0 fiber speed to 100M, card 1 fiber speed to 1000M. root@Moxa:~# mxhsrprpd -t 2 -s 0:0,1:1

Add systemd service to use (if needed)

edit /lib/systemd/system/mx_hsrprp.service

[Unit] Description=Moxa HSR-PRP daemon service [Service] Type=oneshot ExecStart=/usr/sbin/mx_hsrprp start ExecStop=/usr/sbin/mx_hsrprp stop RemainAfterExit=yes [Install] WantedBy=multi-user.target

then enable service.

systemctl enable mx_hsrprp.service

### IRIG-B Utility

Utility for controlling DA-IRIG-B expansion module Compile and install the IRIG-B time sync daemon.

Usage

[root@localhost moxa]# ServiceSyncTime -h Found the IRIG-B module, Hardware ID = 7 IRIG-B time sync daemon. Usage: ServiceSyncTime -t [signal type] -I -i [Time sync interval] -s [Time Source] -p [Parity check mode] -B -t - [signal type] 0 - TTL 1 - DIFF default value is 1 -I - Inverse the input signal -s - [Time Source] The sync source from FREERUN(Internal RTC), Fiber or IRIG-B port 0 - FREERUN(Internal RTC) module 1 - Fiber port 2 - IRIG-B port default value is 2 -i - [Time sync interval] The time interval in seconds to sync the IRIG-B time into system time. 1 ~ 86400 Time sync interval. Default is 10 second. -p - [Parity check mode] Set the parity bit 0: EVEN 1: ODD 2: NONE default value is 0 -B - Run daemon in the background Usage example: Enable to sync time from IRIG-B Port 1, in TTL signal type every 10 seconds. The input signals is not inverse. root@Moxa:~# ServiceSyncTime -t 0 -i 10

Use systemd service step by step

1. Disable NTP service

Warning

NTP service affects IRIG-B service time syncing.

- Disable service

timedatectl set-ntp false

- Make sure NTP service is inactive

timedatectl status Local time: Mon 2023-02-13 02:27:54 PST Universal time: Mon 2023-02-13 10:27:54 UTC RTC time: Mon 2023-02-13 10:27:54 Time zone: America/Los_Angeles (PST, -0800) System clock synchronized: yes NTP service: inactive RTC in local TZ: no

2. Config IRIG-B time sync service

- Edit /usr/sbin/mx_irigb.sh to config service options MX_IRIGB_SERVICESYNCTIME_OPTS.

Info

For more detial about options, please refer to ServiceSyncTime -h

... # The time sync daemon default configure wtih # -t 1 - Sync time in DIFF signal format # -i 10 - The time interval in 10 seconds to sync the IRIG-B time into system time. # -B - Run daemon in the background # MX_IRIGB_SERVICESYNCTIME_OPTS="-t 1 -i 10 -B" ...

3. Start IRIG-B time sync service

- Create and edit systemd service file /lib/systemd/system/mx_irigb.service

[Unit] Description=Moxa DA-IRIG-B daemon service [Service] Type=oneshot ExecStart=/usr/sbin/mx_irigb.sh start ExecStop=/usr/sbin/mx_irigb.sh stop RemainAfterExit=yes [Install] WantedBy=multi-user.target

- Launch service

$ systemctl daemon-reload $ systemctl enable mx_irigb.service Created symlink /etc/systemd/system/multi-user.target.wants/mx_irigb.service → /lib/systemd/system/mx_irigb.service. $ systemctl start mx_irigb.service $ systemctl status mx_irigb.service ● mx_irigb.service - Moxa DA-IRIG-B daemon service Loaded: loaded (/lib/systemd/system/mx_irigb.service; enabled; vendor preset: enabled) Active: active (exited) since Tue 2023-02-14 01:48:29 PST; 5s ago Process: 8322 ExecStart=/usr/sbin/mx_irigb.sh start (code=exited, status=0/SUCCESS) Main PID: 8322 (code=exited, status=0/SUCCESS) CPU: 9ms Feb 14 01:48:29 moxa systemd[1]: Starting Moxa DA-IRIG-B daemon service... Feb 14 01:48:29 moxa systemd[1]: Finished Moxa DA-IRIG-B daemon service.

### MCIM wrapper

MCIM wrapper means Moxa Computer Interface Manager (MCIM) shell script based wrapper. It's provide users with commands similar to MCIM when operating peripherals.

Usage

The Moxa Computer Interface Manager (MCIM) is a tool designed to simplify user control of peripherals. The design of MCIM aims to enhance operational efficiency, enabling users to conveniently handle tasks related to peripheral devices. Usage: mx-interface-mgmt [command] Available Commands: cellular Manages the cellular modem dio Manages digital inputs and outputs for external devices led Manages LED indicators relay Manages the relay mode serialport Manages the serial port input_power Manages the power input state usb_power Manages the usb power state Flags: -h, --help help for mx-interface-mgmt Use "mx-interface-mgmt [command] --help" for more information about a command.

Usage (cellular wrapper)

Usage: mx-interface-mgmt cellular <NAME> <COMMAND> [ARG] Available Commands: Get the power state of a cellular $ mx-interface-mgmt cellular <cellular_name> get_power Set the power state of a cellular $ mx-interface-mgmt cellular <cellular_name> set_power <power_state> Get the SIM slot of a cellular $ mx-interface-mgmt cellular <cellular_name> get_sim_slot Set the SIM slot of a cellular $ mx-interface-mgmt cellular <cellular_name> set_sim_slot <sim_slot> Arguments: cellular_name: The slot number of cellular (e.g. 1¦2) power_state: on¦off sim_slot: 1¦2

Usage (dio wrapper)

Usage: mx-interface-mgmt dio <NAME> <COMMAND> [ARG] Available Commands: Get the state of a dio $ mx-interface-mgmt dio <dio_name> get_state Set the state of a dio $ mx-interface-mgmt dio <dio_name> set_state <dio_state> Arguments: dio_name: The name of dio (e.g. DI0、DO0) dio_state: 0(low)¦1(high)

Usage (led wrapper)

Usage: mx-interface-mgmt led <NAME> <COMMAND> [ARG] Available Commands: Get the state of a LED $ mx-interface-mgmt led <led_name> get_state Set the state of a LED $ mx-interface-mgmt led <led_name> set_state <led_state> Arguments: led_name: The number of LED (e.g. 0, 1, 2, ......) led_state: on¦off

Usage (relay wrapper)

Usage: mx-interface-mgmt relay <NAME> <COMMAND> [ARG] Available Commands: Get the mode of a relay $ mx-interface-mgmt relay <relay_name> get_mode Set the mode of a relay $ mx-interface-mgmt relay <relay_name> set_mode <relay_mode> Arguments: relay_name: The number of relay (e.g. 0, 1, 2, ......) relay_mode: 0¦1 0 --> set to NC (Normal Closed) mode 1 --> set to NO (Normal Open) mode

Usage (input_power wrapper)

Usage: mx-interface-mgmt input_power <NAME> <COMMAND> [ARG] Available Commands: Get the state of a input_power $ mx-interface-mgmt input_power <input_power_name> get_state Arguments: input_power_name: The number of input_power (e.g. 0, 1, 2, ......)

Usage (usb_power wrapper)

Usage: mx-interface-mgmt usb_power <NAME> <COMMAND> [ARG] Available Commands: Get the usb power state of a port $ mx-interface-mgmt usb_power <usb_port> get_state Set the usb power state of a port $ mx-interface-mgmt usb_power <usb_port> set_state <state> Arguments: usb_port: Get USB port power state 0: front 1: rear 2: internal state: Set USB port power state 0: off 1: on

Usage (serialport wrapper)

Usage: mx-interface-mgmt serialport <NAME> <COMMAND> [ARG] Available Commands: Get the interface of a serial port $ mx-interface-mgmt serialport <serialport_name> get_interface Set the interface of a serial port $ mx-interface-mgmt serialport <serialport_name> set_interface <serial_interface> Arguments: serialport_name: The number of serial port (e.g. 0, 1, 2, ......) serial_interface: 0 --> set to RS-232 mode 1 --> set to RS-485-2W mode 2 --> set to RS-422 mode 3 --> set to RS-485-4W mode

## Drivers

### Drivers Applicable Table

| - Available Models | - it87_gpio | - it87_serial | - it87_wdt | - mxuport | - mxu11x0 | - sdhci-pci | - gpio-pca953x | - hid-ft260 | - irigb | - i915 (backport) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - BXP-A100/BXP-C100 | - O | - O | - O | - O | - X | - *[1] | - X | - X | - X | - X |
| - DRP-A100/DRP-C100 | - O | - O | - O | - O | - X | - X | - X | - X | - X | - X |
| - RKP-A110/RKP-C110 | - O | - O | - O | - O | - X | - X | - X | - X | - X | - X |
| - V3200 | - O | - O | - O | - X | - X | - X | - O | - X | - X | - X |
| - V3400 | - O | - O | - O | - X | - X | - X | - O | - X | - X | - X |
| - MPC-3000 | - O | - O | - O | - X | - X | *[2] | - X | - X | - X | - X |
| - DA-820E | - O | - O | - O | - X | - X | - X | - *[3] | - *[4] | - X | - *[5] |
| - DA-820C | - O | - O | - O | - X | - X | - X | - *[3] | - *[4] | - O | - X |
| - DA-682C | - O | - O | - O | - X | - X | - X | - *[3] | - *[4] | - X | - X |
| - DA-681C | - O | - X | - O | - X | - X | - X | - *[3] | - X | - X | - X |
| - DA-680 | - O | - X | - O | - X | - O | - X | - X | - X | - O | - X |
| - RKP-C220 | - O | - O | - O | - O | - X | - X | - X | - X | - X | - X |
| - BXP-A101 | - O | - O | - O | - X | - X | - *[2] | - X | - X | - X | - X |

*[1]: *A100 on Debian 11

*[2]: Debian 11

*[3]: Debian 11, Debian 12, RHEL 9, CentOS 7.9

*[4]: RHEL 9, CentOS 7.9

*[5]: Ubuntu 22.04 LTS

### moxa-it87-gpio-driver

The purpose of moxa-it87-gpio-driver is controlling GPIO interface for IT87xx Super I/O chips, based on Linux kernel drivers/gpio/gpio-it87.c, removed label for Moxa utilities' compatibility and fix-up some issues.

Kernel module information

Once the gpio_it87 driver has been probed, the gpiochip interface /sys/class/gpio/gpiochip* and /sys/class/gpio/gpio* are created by driver.

E.g.

# cat /sys/class/gpio/gpiochip698/label gpio_it87 # cat /sys/class/gpio/gpio699/value 0

Thus, by read/write the gpio value, user can get/set the super IO gpio value.

If Linux kernel version >= 5.x, default uses the libgpiod to set/get set/get gpio value. Alternatively, for Linux kernel version <= 3.x, default uses the sys class gpio to set/get gpio.

### moxa-it87-serial-driver

IT87xx Super I/O chips support six standard serial ports and RS485 automatic direction control (ADDC). This driver provide an interface under misc device for controlling serial register.

Kernel module information

Once the it87_serial driver has been probed, the /sys/class/misc/it87_serial/serial[p] interface are created by driver.

E.g.

# cat /sys/class/misc/it87_serial/serial1/serial1_rs485 0

If returns 0, the RS485 automatic direction control (ADDC) is disabled. Alternatively, if returns 1, the ADDC is enabled.

The UART RS485 ADDC state selection has been imported into mx-uart-ctl utility.

### moxa-it87-wdt-driver

Watchdog timer driver for ITE IT87xx environment control. The moxa-it87-wdt-driver is based on Linux kernel drivers/watchdog/it87_wdt.c driver, and add kernel parameters to support Moxa platform’s hardware design.

Kernel module information

The watchdog device node /dev/watchdog0 is created by it87_wdt driver.

The x86 Linux SDK Wizard will default setup the watchdog daemon configuration file /etc/watchdog.conf and enable service for specific Linux distributions.

Default timeout of watchdog device is 60 seconds (maximum is 65535 seconds). If you want to change timeout value, you need to edit watchdog daemon config file /etc/watchdog.conf

e.g. watchdog timeout for 300 second:

watchdog-timeout = 300

### moxa-mxuport-driver

The purpose of moxa-mxuport-driver is MOXA UPort series driver. This driver remains traditional serial device properties and only dial-in ports will be created.

Kernel module information

The device name for each serial port is /dev/ttyUSBxx which xx is a sequence number maintained by USB subsystem.

The mxuport UART mode selection has been imported into mx-uart-ctl utility.

### moxa-mxu11x0-driver

The purpose of moxa-mxu11x0-driver is Moxa UPort 11x0 USB to Serial Hub driver. The driver can be used in the Linux kernel with the usbcore and usbserial modules.

Kernel module information

The device name for each serial port is /dev/ttyUSBxx which xx is a sequence number maintained by USB subsystem.

The mxu11x0 UART mode selection has been imported into mx-uart-ctl utility.

### moxa-sdhci-pci-driver

Only on BXP-A100 on Debian 11, for resolving the SD card detection issue.

The purpose of moxa-sdhci-pci-driver is SDHCI on PCI bus interface driver.

Due to the SD host controller communicates with the CPU via SDIO, it would not initialize successfully on Debian 11.

Thus to resolve this issue, this driver add module parameter (enable_probe_cd_gpio) to determine probe card detect gpio or not.

modprobe sdhci_pci enable_probe_cd_gpio=0

Or add modprobe configuration file: /lib/modprobe.d/sdhci-pci-option.conf

Kernel message and SD card interface:

### moxa-gpio-pca953x-driver

This driver is for PCA953x 4/8/16/24/40 bit I/O ports control.

Kernel module information

Once the gpio-pca953x driver has been probed, and bind with USB to i2c bridge (e.g. FT260 or CP2112), the gpiochip interface /sys/class/gpio/gpiochip* and /sys/class/gpio/gpio* are created by driver.

The example refers to moxa-it87-gpio-driver section.

### moxa-hid-ft260-driver

This driver is for USB to SMBus master bridge driver on FT260.

Kernel module information

Add Udev Rules to Rebind FT260 Device

To avoid the ft260 hid device is pre-bind to hid-generic subsystem, add udev rules to re-bind to ft260 driver.

Edit /etc/udev/rules.d/11-ft260-pca9535.rules

ACTION=="add", KERNEL=="0003:0403:6030.*", SUBSYSTEM=="hid", DRIVERS=="hid-generic", \ RUN+="/bin/bash -c 'echo $kernel > /sys/bus/hid/drivers/hid-generic/unbind'", \ RUN+="/bin/bash -c 'echo $kernel > /sys/bus/hid/drivers/ft260/bind'"

### moxa-irigb-driver

The IRIG-B driver is for Moxa embedded compute for controlling the IRIG-B device.

Kernel module information

The IRIG-B driver is depends on IRIG-B Utility.

### intel-gpu-i915-backports

Intel® Graphics Driver Backports for Linux® OS (intel-gpu-i915-backports)

Contains the backported kernel module source code of intel GPUs on various OS distributions and LTS Kernels.

Kernel module information

Use lspci -v to check i915 driver is in use

## Libraries

### libgpiod

libgpiod - C library and tools for interacting with the Linux GPIO character device (gpiod stands for GPIO device).

Since Linux kernel 4.8 the GPIO sysfs interface is deprecated. User space should use the character device instead. This library encapsulates the ioctl calls and data structures behind a straightforward API.

The new character device interface guarantees all allocated resources are freed after closing the device file descriptor and adds several new features that are not present in the obsolete sysfs interface.

One device file per gpiochip

- /dev/gpiochip0, /dev/gpiochip1, …, /dev/gpiochipX

Usage:

There are currently six command-line tools available: * gpiodetect - list all gpiochips present on the system, their names, labels and number of GPIO lines * gpioinfo - list all lines of specified gpiochips, their names, consumers, direction, active state and additional flags * gpioget - read values of specified GPIO lines * gpioset - set values of specified GPIO lines, potentially keep the lines exported and wait until timeout, user input or signal * gpiofind - find the gpiochip name and line offset given the line name * gpiomon - wait for events on GPIO lines, specify which events to watch, how many events to process before exiting or if the events should be reported to the console

E.g.

# Read the value of a single GPIO line. $ gpioget gpiochip1 23 0 # Read two values at the same time. Set the active state of the lines to low. $ gpioget --active-low gpiochip1 23 24 1 1 # Set the value of a single line, then exit immediately. # This is useful for floating pins. $ gpioset gpiochip1 23=1

# 5. Basic Linux Concepts

The section introduces basic Linux concepts, like x86 secure boot, IO interfaces, TPM2 module, SD card slot mounting, Linux PTP (IEEE 1588), etc.

To provide skills and basic information for newcomers to learn more about Linux.

## 5.1. Mounting SD Card Slot on BXP/DRP Series

The BXP and DRP series support one SD card slot (SD 3.0 interface (SDHC/SDXC)).

The differences of hardware design between these model are:

- BXP-A100: SDIO interface

- BXP-C100/DRP-A100/DRP-C100: USB to SD Bridge IC (USB2244)

Make sure your SD card is inserted into the SD card slot on your computer, the kernel message should be shown:

For BXP-A100:

To mount the SD Card:

The block devices /dev/mmcblk1, the block device is created from sdhci driver.

Then, user can create a mount point on directory (e.g. /mnt): sudo mount /dev/mmcblk1p1 /mnt

For BXP-C100/DRP-A100/DRP-C100:

The block devices /dev/sdX, where "X" is a letter indicating the specific device (e.g., /dev/sdb, /dev/sdc, etc.).

Then, user can create a mount point on directory (e.g. /mnt): sudo mount /dev/sdX /mnt

## 5.2. Secure Boot

The UEFI Secure Boot is a security feature that has been widely adopted in modern computer systems, especially those running Windows and some Linux distributions.

Its primary purpose is to ensure the integrity and authenticity of the operating system and bootloader during the system boot process, protecting the system against boot-time malware and other unauthorized software.

Secure Boot Purpose

Secure Boot is designed to prevent the loading of malicious software, such as rootkits and bootkits, during the boot process.

It does this by ensuring that only trusted and digitally signed bootloaders and OS kernels are executed.

Thus, if user loads unsigned bootloaders and OS kernels on target Linux distributions when UEFI secure boot has been enabled on BIOS menu, the boot process or kernel modules should be failed due to unauthorized policy.

Operating System Support

User can be considered to refer to the following website links for more UEFI secure boot information.

- Debian Secure Boot

- Ubuntu Secure Boot

- RedHat Secure Boot

## 5.3. Linux PTP (IEEE 1588)

The Precision Time Protocol (PTP) is a protocol used to synchronize clocks throughout a computer network. PTP provides higher precision and faster synchronization than NTP even without hardware support. With hardware support, sub-microsecond accuracy can be expected.

Whereas NTP is intended for WAN use, PTP is designed for LAN environments and makes use of UDP multicast.

Available LAN chip

- Intel I210 (driver: igb)

- Intel I219 (driver: e1000e)

Debian Linuxptp package

Linuxptp package is an implementation of the Precision Time Protocol (PTP) according to IEEE standard 1588 for Debian Linux. Features include:

- support for hardware and software time stamping via the Linux SO_TIMESTAMPING socket option.

- support for the Linux PTP Hardware Clock (PHC) subsystem by using the clock_gettime family of calls, including the new clock_adjtimex system call

- implementation of Boundary Clock (BC) and Ordinary Clock (OC)

- transport over UDP/IPv4, UDP/IPv6, and raw Ethernet (Layer 2)

- support for IEEE 802.1AS-2011 in the role of end station

Debian phc2sys program

phc2sys is a program which synchronizes two or more clocks in the system. Typically, it is used to synchronize the system clock to a PTP hardware clock (PHC), which itself is synchronized by the ptp4l(8) program. See manpage for more information.

- Prerequisite

  - Install Debian 11 or later version

  - Install Linuxptp package: apt update && apt install linuxptp

  - Stop and disable systemd time sync daemon service to avoid some unexpected operations: systemctl stop systemd-timesyncd && systemctl disable systemd-timesyncd

### Example for Linux PTP setting up

#### [Ordinary Clock (OC) mode]

Set as OC master mode: Layer 2, P2P mode, peer delay mechanism

# Assume A side interface device is 'enp4s0' ip link set dev enp4s0 up ptp4l -m -2 -P -i enp4s0

Set as OC slave mode: Layer 2, P2P mode, peer delay mechanism

# Assume B side interface device is 'enp5s0' ip link set dev enp5s0 up ptp4l -m -2 -P -s -i enp5s0 # or with log: ptp4l -m -2 -s -P -i enp5s0 2>&1 ¦ tee $(date +%Y%m%d%H%M%S.log) # use phc2sys to sync sys clock for 10Hz phc2sys -a -m -r -R 10

#### [Boundary Clock (BC) mode]

Set as BC mode host

- clock_type Specifies the kind of PTP clock. Valid values are "OC" for ordinary clock, "BC" for boundary clock, "P2P_TC" for peer to peer transparent clock, and "E2E_TC" for end to end transparent clock. An multi-port ordinary clock will automatically be configured as a boundary clock. The default is "OC".

- boundary_clock_jbod When running as a boundary clock (that is, when more than one network interface is configured), ptp4l performs a sanity check to make sure that all of the ports share the same hardware clock device. This option allows ptp4l to work as a boundary clock using "just a bunch of devices" that are not synchronized to each other. For this mode, the collection of clocks must be synchronized by an external program, for example phc2sys(8) in "automatic" mode. The default is 0 (disabled).

Example for BC mode

# For example, edit config file 'bc.cfg' # and assume 'enp12s0' and 'enp4s0' are connected network interface [global] sanity_freq_limit 0 step_threshold 0.000002 tx_timestamp_timeout 10 logMinPdelayReqInterval 0 logSyncInterval 0 logAnnounceInterval 0 announceReceiptTimeout 3 syncReceiptTimeout 2 twoStepFlag 1 summary_interval 0 clock_type BC priority1 128 priority2 127 delay_mechanism P2P [enp12s0] boundary_clock_jbod 1 network_transport L2 fault_reset_interval 0 [enp4s0] boundary_clock_jbod 1 network_transport L2 fault_reset_interval 0 # run the ptp4l procedure ip link set dev enp12s0 up ip link set dev enp4s0 up ptp4l -m -f bc.cfg # use phc2sys to sync sys clock for 10Hz phc2sys -a -m -r -R 10

On OC Grandmaster

# assume interface is enp5s0 ip link set dev enp5s0 up ptp4l -2 -m -P -i enp5s0

On OC Slave

# assume interface is enp4s0 ip link set dev enp4s0 up ptp4l -2 -m -s -P -i enp4s0 # with log: ptp4l -2 -m -s -P -i enp4s0 2>&1 ¦ tee $(date +%Y%m%d%H%M%S.log)

#### [Transparent Clock (TC) mode]

Set TC mode host

# For example, edit config file 'tc.cfg' # and assume 'enp12s0' and 'enp4s0' are connected network interface [global] priority1 254 priority2 253 free_running 1 freq_est_interval 3 tc_spanning_tree 1 clock_type P2P_TC network_transport L2 delay_mechanism P2P [enp12s0] egressLatency 0 ingressLatency 0 delay_mechanism P2P network_transport L2 [enp4s0] egressLatency 0 ingressLatency 0 delay_mechanism P2P network_transport L2 # run the ptp4l procedure ip link set dev enp12s0 up ip link set dev enp4s0 up ptp4l -m -f tc.cfg # use phc2sys to sync sys clock between master & slave for 10Hz # -c Specify the slave clock by device (e.g. /dev/ptp1) or interface (e.g. eth1) # -s Specify the master clock by device (e.g. /dev/ptp0) or interface (e.g. eth0) phc2sys -s enp12s0 -c enp4s0 -O 0 -R 10 -m

As OC Grandmaster

# assume interface is enp5s0 ip link set dev enp5s0 up ptp4l -2 -m -P -i enp5s0

As OC Slave

# assume interface is enp4s0 ip link set dev enp4s0 up ptp4l -2 -m -s -P -i enp4s0 # use phc2sys to sync sys clock for 10Hz on slve phc2sys -a -m -r -R 10

# 6. Troubleshooting

The troubleshooting section provides fundamental skills for system logging, debugging, the debug of Moxa x86 SDK Wizard and issues tracing.

## 6.1. How to Print Kernel Message from Linux Environment

The dmesg command is used to display the kernel ring buffer, which contains messages related to the kernel and hardware events.

It's a useful tool for troubleshooting hardware-related issues, monitoring system-level events and diagnosing hardware issues.

To simply view the kernel ring buffer, run the following command: dmesg

You can save the output of dmesg to a file for further analysis. For instance, to save the log to a file named kernel.log, use the following command:

# save kernel message to log dmesg >kernel.log # or simply to save the error and warninglevel log: dmesg --level=err,warn > kernel_err_warn.log

## 6.2. How to Collect Systems Logs from Linux Environment

The following procedure describes the collecting of log files. Log files in the /var/log directory.

Archive and compress all log files and put them in /tmp

tar czvf /tmp/varlog.tar.gz /var/log/*.log.*

The output file /tmp/varlog.tar.gz can be transferred for debugging usage.

## 6.3. How to Get Installation Logs from Moxa x86 Linux SDK Install Wizard

Moxa x86 Linux SDK provides self-test for diagnosing the status of drivers and tools after installation. To simply see the log, run the following command:

./install.sh --selftest

Then the self test cases will check the SDK status and print on terminal, for example:

For further, the log of installation is also created on Moxa_x86_Linux_Install_Wizard_<version>_Build_<build_date>/install.log

User can consider to view the log file and check issues.

## 6.4. How to Get Hardware Information on Host

IOS exports the hardware information on DMI (Desktop Management Interface) table.

Linux dmidecode is a tool for dumping a computer DMI (some say SMBIOS) table contents in a human-readable format. This table contains a description of the system’s hardware components, as well as other useful pieces of information such as serial numbers and BIOS revision.

Install dmidecode Package

- Ubuntu/Debian: sudo apt-get install dmidecode

- RHEL: sudo yum install dmidecode

Example

[Get model name and hardware version]

The Option 1 (or Option 2) displays the 16 bytes information, for example: RKP A110000091

RKP A110000091 means

- PCBA name = RKP

- PCBA number = A110

- PCBA serial = 0

- PCBA type = 00

- PCBA hw version = 091 (v0.91)

How to get information from dmitable

# dmidecode -t 12 Handle 0x0021, DMI type 12, 5 bytes System Configuration Options Option 1: RKP A110000091 Option 2: Option 3: ...

[Get current BIOS version]

# dmidecode -t bios BIOS Information Vendor: INSYDE Corp. Version: V1.0.0S04 Release Date: 05/15/2023 Address: 0xE0000 Runtime Size: 128 kB ROM Size: 10 MB ...

[Get memory and processor hardware information]

# dmidecode -t memory Physical Memory Array Location: System Board Or Motherboard Use: System Memory Error Correction Type: None Maximum Capacity: 16 GB Error Information Handle: Not Provided Number Of Devices: 2 ... # sudo dmidecode -t processor Processor Information Socket Designation: U3E1 Type: Central Processor Family: Other Manufacturer: Intel(R) Corporation ID: 61 06 09 00 FF FB EB BF Version: Intel Atom(R) x6425E Processor @ 2.00GHz Voltage: 1.1 V External Clock: 100 MHz ...

# 7. Appendix

## 7.1. The License/Commercial-Use of Linux Distributions

A Linux distribution is a version of the Linux operating system that includes the Linux kernel, system utilities, libraries, and additional software and applications. Linux distributions are created by various organizations, communities, and individuals, each tailoring the operating system to meet specific needs and preferences.

Linux distribution include:

### Debian

Debian is a free and open-source operating system, and its intellectual property rights policy is based on a commitment to free software principles. Debian adheres to a set of guidelines and policies outlined in the Debian Free Software Guidelines (DFSG). The DFSG defines the criteria that software must meet to be considered "free" in the context of Debian.

Commercial use:

Free redistribution.

The license of a Debian component may not restrict any party from selling or giving away the software as a component of an aggregate software distribution containing programs from several different sources. The license may not require a royalty or other fee for such sale.

https://wiki.debian.org/DebianFreeSoftwareGuidelines

https://www.debian.org/trademark

https://wiki.debian.org/ProposedTrademarkPolicy

### Ubuntu

Ubuntu is built by Canonical and the Ubuntu community. We share access rights owned by Canonical with the Ubuntu community for the purposes of discussion, development and advocacy. We recognise that most of the open source discussion and development areas are for non-commercial purposes and we therefore allow the use of Canonical IP in this context, as long as there is no commercial use and that the Canonical IP is used in accordance with this IPRights Policy.

You can modify Ubuntu for personal or internal commercial use.

You can redistribute Ubuntu, but only where there has been no modification to it.

For more Canonical’s intellectual property rights policy:

https://ubuntu.com/legal/intellectual-property-policy

### Red Hat Enterprise Linux (RHEL)

Red Hat Enterprise Linux (RHEL) is a commercial Linux distribution provided by Red Hat, Inc. It is designed for enterprise environments and comes with a subscription-based pricing model.

https://www.redhat.com/en/store/linux-platforms

https://www.redhat.com/en/about/trademark-guidelines-and-policies

https://www.redhat.com/en/about/terms-use

### CentOS

The CentOS Linux and CentOS Stream distributions are compilations of software packages. Each package is governed by its own license. The CentOS Linux and CentOS Stream compilation copyright is licensed under GPLv2. To the extent you hold any copyright in the selection, coordination, or arrangement of packages making up the CentOS Linux or CentOS Stream distributions, you license that copyright under GPLv2.

https://www.centos.org/legal/licensing-policy/

# 8. Revision History

| - Version | - Date | - Author | - Revision Notes |
| --- | --- | --- | --- |
| - 2.0 | - 2024/03/12 | - Elvis CW Yao | - First created. |
| - 2.1 | - 2024/09/05 | Elvis CW Yao Wilson YS Huang Henry LC Chen | - Add the following model support - V3200 series - MPC-3000 series - DA-820E series - DA-820C series - DA-682C series - DA-681C series - DA-680 series - Add Debian 12, Ubuntu 20.04 into Linux OS list - Add LTE (mPCIe slot) Module Control section - Add HSR/PRP Utility section - Add IRIG-B Utility section - Add MCIM wrapper section - Add Scaler Utility section - Add MCU upgrade tool section - Add moxa-gpio-pca953x-driver section - Add moxa-mxu11x0-driver section - Add moxa-hid-ft260-driver section - Add moxa-irigb-driver section - Add intel-gpu-i915-backports section |
| - 2.2 | - 2024/11/12 | Elvis CW Yao - Wilson YS Huang | - Add the following model support - V3400 series - BXP-A101 series - Add disk hotswap daemon - Add Moxa MCU Manager |